HACK3D Damaged G-code Identification

Geometry and Filament Evidence for Pawn, Bishop, and Queen

Research question: Which reference piece best explains the preserved geometry and the missing extrusion in the damaged G-code?

Figure 1. Reference candidates used in the analysis: Pawn, Bishop, and Queen.

Evidence Pipeline

| Stage | Input | Output |
| --- | --- | --- |
| 1 | damaged_chess.gcode | Clean deposition segments |
| 2 | Deposition segments | Layer profile: Z -> half-width |
| 3 | Reference image | Pawn / Bishop / Queen profiles |
| 4 | Profile comparison | RMSE / MAE / max error |
| 5 | Extrusion metadata + learned E/path | Predicted missing filament |

Decision rule: Use geometry as one evidence source and filament consumption as a second, independent cross-check.

1. G-code Cleaning and Measured Damaged Geometry

Parsed motion state from G0/G1, G90/G91, M82/M83, and G92.

Kept only XY moves with positive extrusion (deposition).

Grouped deposition by Z layer and measured the outer X/Y extent.

Converted the toolpath centerline to a surface half-width using the extrusion-width offset.

| Measured item | Result |
| --- | --- |
| Clean deposition segments | 27,666 |
| Printing layers | 237 |
| Printing Z range | 0.35 to 23.95 mm |
| Parsed deposition filament | 3198.14 mm |

2. Reference Profile Extraction

Figure 2. Contour check. Blue lines mark piece centerlines; red traces show the extracted reference boundary.

Reference heights: Pawn 45 mm, Bishop 60 mm, Queen 70 mm.

Image coordinates were scaled to millimeters.

Each candidate was represented as Z -> half-width.

Geometry comparison used only the preserved range up to Z = 23.95 mm.

3. Geometry Match

Metric: RMSE between the damaged half-width profile and each reference profile at matched Z positions.

| Candidate | RMSE (mm) | MAE (mm) | Max error (mm) |
| --- | --- | --- | --- |
| Bishop | 0.1040 | 0.0570 | 0.6510 |
| Queen | 0.1882 | 0.1212 | 0.7088 |
| Pawn | 1.4866 | 1.2178 | 2.2047 |

Geometry result

Bishop is the best lower-profile match: RMSE = 0.1040 mm.

Queen is also close: RMSE = 0.1882 mm.

Pawn is clearly inconsistent: RMSE = 1.4866 mm.

Sensitivity Check

| Parameter sweep | Bishop wins | Queen wins | Pawn wins |
| --- | --- | --- | --- |
| Full preserved profile | 78.3% | 21.7% | 0% |
| Body-only profile | 83.7% | 16.3% | 0% |

Interpretation: The preserved lower geometry favors Bishop, but the Bishop-Queen separation is small enough that geometry alone is not decisive.

4. Missing-Filament Test

Observed missing extrusion budget: 4290.7 - 3198.14 = 1092.56 mm

Figure 3. Predicted missing filament for the three candidates. The dashed line is the observed missing budget.

| Candidate | Added layers | Predicted (mm) | Abs. error (mm) | Rel. error |
| --- | --- | --- | --- | --- |
| Queen | 460 | 1124.07 | 31.50 | 2.88% |
| Bishop | 360 | 825.32 | 267.25 | 24.46% |
| Pawn | 210 | 528.45 | 564.11 | 51.63% |

5. Conclusion

Geometry evidence: Bishop is the closest match to the preserved lower shape.

Filament evidence: Queen is much closer to the observed missing budget.

Queen prediction: 1124.07 mm vs. 1092.56 mm observed (2.88% error).

Bishop prediction error: 24.46%; Pawn prediction error: 51.63%.

Combined evidence therefore favors the Queen as the most plausible target.

Limitations

Reference geometry was measured from a raster image, so pixel calibration introduces uncertainty.

Only the lower 23.95 mm of the damaged piece is directly preserved.

Filament prediction is a model-based consistency check, not proof of byte-for-byte recovery.